ASSIGNMENT#2

Computer Network

Name: Muhammad Hassan Siddiqui        Roll no: 24K-1051      Section: BCS-5A      Instructor: Sir Ghufran

Question#1

Scenario: After a major earthquake, a disaster-response organization links its Central Emergency Operations Center (EOC) to field rescue teams over a satellite link (R = 2 Mbps, RTT = 500 ms, packet size L = 1,000 bytes = 8,000 bits). The link loses, corrupts and delays packets and ACKs, and bandwidth is limited. Teams send casualty lists and supply requests; the EOC sends rescue assignments. A lost or corrupted packet could send a team to the wrong location, so every packet must arrive correctly and in order. You must design and test the transport protocol for this network.

- Why is reliable data transfer important in this network?

Reliable data transfer is critical because the network carries casualty lists, supply requests and rescue assignments, where a single lost or corrupted packet could send a team to the wrong location. The satellite link loses, corrupts and delays both packets and ACKs, so the underlying channel cannot be trusted. The transport protocol must therefore detect errors (checksums), preserve ordering (sequence numbers) and recover from loss (ACKs, timeouts and retransmissions) so that every packet arrives correctly and in order.

- Why can Stop-and-Wait be inefficient on this link?

Stop-and-Wait is inefficient here because the sender transmits one packet and then must wait for its ACK before sending the next. On this link, sending a packet takes only L/R = 4 ms, but the satellite RTT is 500 ms, so the sender is idle almost all the time. The utilization is U = 0.004 / (0.5 + 0.004) ≈ 0.8%, which wastes nearly all of the limited 2 Mbps bandwidth, and any lost packet or ACK adds a further timeout delay.

- Why is pipelining useful here?

Pipelining is useful here because the 500 ms RTT is far longer than the 4 ms needed to transmit one packet, so a Stop-and-Wait sender would sit idle almost all the time. With pipelining, the sender transmits several packets (a window of N) before the first ACK returns, keeping the link busy during the wait. For N = 4, utilization rises from about 0.8% to about 3.2%, so critical data such as casualty lists and rescue assignments is delivered faster over the limited bandwidth.

- Is GBN or SR more suitable for this scenario? Give two technical reasons.

SR is more suitable for this scenario for two reasons. First, bandwidth is limited to 2 Mbps, and under GBN a single loss forces the sender to retransmit every outstanding packet, while SR retransmits only the missing one, saving scarce capacity. Second, with a 500 ms RTT and a channel that delays packets and ACKs, premature timeouts and out-of-order arrivals are common: GBN discards out-of-order packets and resends the whole window, whereas SR buffers them and retransmits only what is actually missing, while still delivering data in order.

Question#2

(a) Go-Back-N: show packet transmissions, ACKs, receiver behaviour for out-of-order packets, the timeout, retransmitted packets and sender window movement.

| Event | Sender | Packet | Receiver | ACK | Window |
| --- | --- | --- | --- | --- | --- |
| 1 | Send pkt 0 (start timer) | 0 | - | - | [0-3], nextseqnum=1 |
| 2 | Send pkt 1 | 1 | - | - | [0-3], nextseqnum=2 |
| 3 | Send pkt 2 | 2, LOST | - | - | [0-3], nextseqnum=3 |
| 4 | Send pkt 3 | 3 | - | - | [0-3], nextseqnum=4 (full) |
| 5 | - | 0 | Deliver pkt 0, expectedseqnum=1 | ACK0 | - |
| 6 | Receive ACK0, send pkt 4 | 4 | - | - | base=1 → [1-4] |
| 7 | - | 1 | Deliver pkt 1, expectedseqnum=2 | ACK1 | - |
| 8 | Receive ACK1, send pkt 5 | 5 | - | - | base=2 → [2-5] |
| 9 | - | 3 | Out of order (expected 2), discard | ACK1 (duplicate) | - |
| 10 | - | 4 | Out of order, discard | ACK1 | - |
| 11 | - | 5 | Out of order, discard | ACK1 | - |
| 12 | Receive duplicate ACK1s, ignore | - | - | - | base stays 2, [2-5] |
| 13 | Timeout on pkt 2 (base), go back and resend 2, 3, 4, 5 | 2, 3, 4, 5 | - | - | [2-5] |
| 14 | - | 2 | Deliver pkt 2, expectedseqnum=3 | ACK2 | - |
| 15 | - | 3 | Deliver pkt 3, expectedseqnum=4 | ACK3 | - |
| 16 | - | 4 | Deliver pkt 4, expectedseqnum=5 | ACK4 | - |
| 17 | - | 5 | Deliver pkt 5, expectedseqnum=6 | ACK5 | - |
| 18 | Receive ACK2, send pkt 6 | 6 | - | - | base=3 → [3-6] |
| 19 | Receive ACK3, send pkt 7 | 7 | - | - | base=4 → [4-7] |
| 20 | Receive ACK4, send pkt 8 | 8 | - | - | base=5 → [5-8] |
| 21 | Receive ACK5, send pkt 9 | 9 | - | - | base=6 → [6-9] |
| 22 | - | 6, 7, 8, 9 | Deliver each, expectedseqnum=10 | ACK6-ACK9 | - |
| 23 | Receive ACK6-ACK9, stop timer | - | - | - | base=10, all delivered |

(b) Selective Repeat: show packet transmissions, individual ACKs, receiver buffering, the timeout, retransmitted packet(s) and sender/receiver window movement.

| Event | Sender | Packet | Receiver | ACK | Sender window / Receiver window |
| --- | --- | --- | --- | --- | --- |
| 1 | Send pkt 0 (timer 0) | 0 | - | - | S [0-3] / R [0-3] |
| 2 | Send pkt 1 (timer 1) | 1 | - | - | S [0-3] |
| 3 | Send pkt 2 (timer 2) | 2, LOST | - | - | S [0-3] |
| 4 | Send pkt 3 (timer 3) | 3 | - | - | S [0-3] (full) |
| 5 | - | 0 | Deliver pkt 0, rcv_base=1 | ACK0 | R [1-4] |
| 6 | Receive ACK0, mark 0 ACKed, send pkt 4 | 4 | - | - | S [1-4] |
| 7 | - | 1 | Deliver pkt 1, rcv_base=2 | ACK1 | R [2-5] |
| 8 | Receive ACK1, mark 1 ACKed, send pkt 5 | 5 | - | - | S [2-5] |
| 9 | - | 3 | In window, buffer pkt 3 (rcv_base stays 2) | ACK3 | R [2-5] |
| 10 | - | 4 | Buffer pkt 4 | ACK4 | R [2-5] |
| 11 | - | 5 | Buffer pkt 5 | ACK5 | R [2-5] |
| 12 | Receive ACK3, ACK4, ACK5, mark 3, 4, 5 ACKed | - | - | - | S [2-5] (send_base stuck at 2) |
| 13 | Timeout on pkt 2 only, resend pkt 2 | 2 | - | - | S [2-5] |
| 14 | - | 2 | Deliver 2, then buffered 3, 4, 5 in order, rcv_base=6 | ACK2 | R [6-9] |
| 15 | Receive ACK2, send_base jumps to 6 (3, 4, 5 already ACKed), send 6, 7, 8, 9 | 6, 7, 8, 9 | - | - | S [6-9] |
| 16 | - | 6, 7, 8, 9 | Deliver each, rcv_base=10 | ACK6-ACK9 | R window past end |
| 17 | Receive ACK6-ACK9 | - | - | - | send_base=10, all delivered |

Finally, answer: How many packets are retransmitted in GBN and in SR in this scenario, and why are they different?

Total GBN transmissions: 10 original + 4 retransmissions = 14

Total SR transmissions: 10 original + 1 retransmission = 11

### Retransmission comparison

- GBN retransmits 4 packets (2, 3, 4, 5)

- SR retransmits 1 packet (2)

Why they differ:

- The GBN receiver discards out-of-order packets and only sends cumulative ACKs, so the sender sees ACK1 repeated and has no idea that 3, 4 and 5 arrived. After the timeout it must resend everything from base onward.

- The SR receiver buffers out-of-order packets and sends individual ACKs, so the sender knows exactly which packets are missing. Only pkt 2’s timer expires, so only pkt 2 is resent, and the buffered packets are delivered in order the moment it arrives.

Question#3

This shows the reliability of the channel since it shows verified: True

I made packet 2 lost for GBN first, Now lets do for SR.

There we did for the SR.

Used my roll number to setup the seed.

Setting up the final results.

Github repo:

https://github.com/MuhammadHassanSiddiqui123/gbn-vs-sr-simulator

Question#4

Result table:

With a reliable channel (A), GBN and SR behave identically, since neither needs to retransmit. Under loss (B) and mixed errors (C), SR retransmits far fewer packets (53 vs 127 and 42 vs 144) because it resends only the packet that timed out, while GBN must resend the whole outstanding window after one loss. GBN’s receiver also discards every out-of-order packet, so delayed or reordered packets in scenario C cause extra retransmissions that SR avoids by buffering them. As a result SR completes the transfer sooner and achieves higher throughput in the error scenarios, at the cost of a more complex sender and receiver.

Question#5

Part A:

Stop-and-Wait sends one packet and then idles until its ACK returns, so the sender is busy for only 4 ms out of every 504 ms, giving U ≈ 0.79%. Pipelining lets the sender transmit N packets before the first ACK arrives, so the busy time per cycle is N × L/R. With N = 4, U = 4 × 0.004 / 0.504 ≈ 3.17%, about four times better. Utilization grows linearly with N until the pipe is full.

Part B:

- Top panel (N = 5, unsafe): the sender sent packets 0–4 and every ACK was lost. The receiver has delivered them, so its window moves on to {5, 6, 7, 0, 1}. Seq numbers 0 and 1 (red) now belong to both windows. When the sender times out and resends the old packet 0, the receiver sees seq 0 inside its window and accepts it as a new packet.

- Bottom panel (N = 4, safe): the receiver’s window moves to {4, 5, 6, 7}, which shares no numbers with the sender’s {0, 1, 2, 3}. The retransmitted seq 0 is outside the window, so the receiver knows it is old, re-sends the ACK and discards it.